This training is to be completed per OSHA 29 CFR Subpart I 1910.1200 – Hazard Communication Standard in order to train employees about the Hazardous Chemicals they may be exposed to while working at this site.

This Training is to be used in addition to the training received in Safeschools and New Hire Orientation Course.  These courses contain details on:

- The requirements of the Hazard Communication Standard, 29 CFR 1910.1200

- The details of the Hazcom program, including: explanation of the labeling system and general information on the components of a material safety data sheet (SDS).

### Site-Specific Hazard Communication Plan and SDS Locations:

The site-specific Hazard Communication Plan is Located:

Material Safety Data Sheet (SDS) binders are located: < insert site-specific information in the table below>

| Department | Location |
| --- | --- |
| Housekeeping <example> | Outside of Site Facility Coordinator Office <example> |

Department/Task Specific Hazardous Chemical Exposures:

List all chemicals on page 2 that are reviewed with the employee, using the associated SDS sheets, ensuring that all of the information in the table below is reviewed, at a minimum.  Perform review for each chemical that the employee may be exposed to during a normal workday.

| Topics Required to Cover on SDS | Examples of the SDS discussion points for Isopropyl Alcohol |  |  |  |
| --- | --- | --- | --- | --- |
| Chemical Name: | Isopropyl Alcohol |  |  |  |
| Department/Task Used: | (Maintenance). |  |  |  |
| Methods of Detecting a Release: | Colorless liquid with the odor of rubbing alcohol. |  |  |  |
| Hazards: | Class 1B Flammable Liquid |  |  |  |
| Symptoms of Exposure: | Irritant to the eyes, nose, throat; drowsiness, dizziness, headache; dry cracking skin; in animals: Narcosis. |  |  |  |
| Personal Protection <Review PPE Requirement for Hazard | Respiratory | Body | Hand | Eye |
|  | Req. Dependant on IH Exposure Assessments | Use chemical apron and arm guards if this exposure exists | Neoprene, Nitrile, PVC, Viton, or Natural Rubber Gloves | Chemical Safety Goggles |
| Safe Use Procedures: | Use only with non-sparking type tools and equipment. Never use near hot work operations. |  |  |  |
| First Aid Measures: | Inhalation | Ingestion | Skin Contact | Eye Contact |
|  | Remove to fresh air, seek medical attention | Drink large amounts of water – get medical attention. | Immediately flush skin with plenty of water for at least 15 minutes. |  |

List of chemicals that were reviewed using the above criteria (Attached additional pages as necessary):

| Chemical Name: | Department/Task Where Used | Chemical Reviewed With Employee Using the Associated SDS?(Check box if ‘Yes’) |
| --- | --- | --- |
| Isopropyl Alcohol | Glass cleaner in offices |  |

Hazardous Non-Routine Tasks:

If the employee is responsible for performing any Hazardous Non-Routine Tasks, perform a review of each task with the employee using the completed form(s) in the Hazard Communications procedure or equivalent.

| Date of Training: |  |  |
| --- | --- | --- |
|  | Print | Sign |
| Name of Person Receiving Training: |  |  |
| Name of Trainer: |  |  |